# Test Beam Module Spreadsheet Description

## Definitions

| Global Channel | Channel number from 0 to 14*128-1. Global channel 0 is on the phi side, chip 0, channel 0. |
| --- | --- |
| Chip | Chip number from 0 to 13. 0-6 are on the phi side. 7-13 are on the z side (Chip 7 is elctronics address 0 on the z side). |
| Channel | Channel number from 0-127 on the chip. |
| Side | Phi (n) or Z (p) side of silicon. |
| Silicon Strip | Strip number on the silicon. Numbering starts from 1, not 0. On the z side, strips 1-441 are on wafer 1 (wafer closest to HDI) and strips 442-882 are on wafer 2. |
| DFA Fault | Code for the fault type from DFA testing (see below). |
| Bonding | Code for wire bonding (see below). |
| HDI Fault, Unbonded | Code for HDI faults found before wirebonding. |
| HDI Fault, Bonded | Code for HDI faults found after bonding. |
| Gains, Unbonded | Channel gains in mV/fC measured before bonding to DFA. |
| Intercepts, Unbonded | Channel intercepts in mV measured before bonding to DFA. |
| Gains, Bonded | Channel gains in mV/fC measured after bonding to DFA. |
| Intercepts, Bonded | Channel gains in mV measured after bonding to DFA. |
| Noise (counts) | Channel noise, measured after bonding, in units of threshold DAC counts. |
| Noise (electrons) | Channel noise, measured after bonding, in units of ENC. This is just Noise(counts)*6250*4.29/(Gain, Bonded) |

## DFA Fault Codes

| 1 | Pinhole |
| --- | --- |
| 2 | Short between readout strips |
| 3 | Short between readout strip and floating strip |
| 4 | Short to p-stop |
| 5 | Open on Upilex |
| 6 | Broken metal on Silicon |
| 666 | Unknown DFA |

## Bonding Codes

| 1 | No Bond, Silicon-Silicon |
| --- | --- |
| 2 | No Bond, Silicon-Upilex |
| 3 | No Bond, Upilex-HDI |
| 4 | No Bond, plucked for study after gain and noise measurement |

## HDI Fault Codes

| 1 | Dead Channel |
| --- | --- |
| 2 | Noisy Channel |
| 3 | Inefficient Channel |
| 4 | Digital Problem(turns on even when masked off) |
| 5 | Stuck On |
| 777 | Flaky, unknown problem |

For an explanation of how the gains, offsets and noise are measured, see HREF="http://www.hep.ucsb.edu/BaBar/gains.html.  In the spreadsheet we use channel-by-channel gains rather than an average value.

Note:  A problem developed on Chip 6 between the unbonded and bonded tests.  It is now effectively dead.  Also, the extraction of gains and offset for chips 3 and 11 could be suspect due to intermittent problems.  See HREF="http://www.hep.ucsb.edu/BaBar/TestBeamHints.html.